IBM ThinkPad 701                                      September 14, 1995

System Program Service Diskette

======================================================

What This Diskette Does

-----------------------

This System Program Service diskette updates the system program stored

in ROM of the ThinkPad 701C and 701CS.

If you are experiencing one of the problems listed in the following

Summary of Problem Fixes section, please follow the instructions given

in the Updating FLASH ROM section below to update the system program

of the above-listed ThinkPad computer.

Current Levels

--------------

The latest level of Diagnostics is 1F.

The latest level of BIOS is        3M.

BIOS Changes/Fixes

------------------

3M

Fixed

-----

Closed all PCMCIA IO & Memory windows in POST so that warm boot

will not leave phantom COM ports.

Will power off if eject is requested and an external video card

is present.

Reset the Manufacturing Diskette everytime through POST.

Saved/Restored the BitMask Register & Set/Reset Enable Register

so that Japanese video modes are not corrupted after a resume.

New/Changed

-----------

Changed Win95 PnP code to meet Win95 logo requirements by eliminating

the reporting of DMA on the ECP parallel ports

Changed the way devices are reported through PnP. A new device node

is only created if the address or IRQ of a device changes

from docked to undocked or vice versa.

Added new TrackPoint III option 'Auto' that will automatically enable

or disable the TP III based on whether an external mouse

is attached.

Enabled the ability to hibernate to an existing HPFS hibernate file.

3J

Fixed

-----

The century byte will be correctly updated when an alarm time is

set in the Configuration Utility.

The DOS DATE command and Windows 3.1 Date/Time will now return a

the correct century if the century rollover occurred while

the machine was powered on.

The docking station was reporting that the dock was locked on

an operating system eject request. This has been corrected.

PCMCIA cards that are not powered but are in the PCMCIA slot were

causing the PCMCIA controller to stay awake in suspend. This

has been corrected. Adding $ICPMDOS.SYS or $ICPMOS2.SYS

will power off the card properly if APM is running.

PCMCIA controller was being suspended even though a powered card

was in a slot if the box was suspended on AC and then the

AC power was removed. This has been corrected.

Accesses to the FDD were locking up systems for mode 3 drives (Japan).

Low memory was being corrupted. This has been fixed. The

memory was also being corrupted if the FDD returned a bad

return code.

Changed Plug & Play BIOS to meet Win95 requirements.

Fixed some Win95 problems in our Plug & Play BIOS.

New/Changed

-----------

Keyboard initialization time has been shortened.

Video cards are now allowed in the DOCK II. Note: if one is installed,

most Fn keys are disabled. Only Mute, Volume Up, Volume Down,

and Stop HDD work, but no icon is displayed. Suspending,

hibernating, accessing the Configuration Utility, and other

functions that required a change to the video are disabled.

3I

Fixed

-----

The Brazilian MRI did not match the User's Guide

Garbage appeared on the STN screen when in simultaneous display

at elevated temperatures.

Rebooting under Win95 caused serial port errors

If system docked and user selects 'Eject' from the docking message,

the system will now eject properly.

New/Changed

-----------

Added a function to update the CMOS century byte when the system is

resumed from suspend or turned on from power off.

Changed Standby (Fn+F3) so that it does not stop the clock waiting

for an interrupt, but rather rely on APM to stop the clock.

3H

New/Changed

-----------

Added Plug & Play support

Added support for IBM Dock II

Diagnostics Changes/Fixes

-------------------------

*************************************************************************

Updating your ThinkPad 701 FLASH ROM (BIOS and Diags)

-----------------------------------------------------

Your ThinkPad 701 has been designed so BIOS and Diagnostics upgrades

can be done by you, the end user (quickly and easily).  There is no

need to change anything on your hard drive as a result of this upgrade,

and your applications will work as they did before.

This update is only for the ThinkPad 701 (all models):

1.  Print these instructions or display them on another computer.

2.  Enter the Configuration Utility by pressing Fn-F1.  Page Down once

to page 2 of the menu.  Check "Startup Sequence".  It should be

set to "Smart" or "Conventional", but not "Hard Disk".

Also make note of any customizing you have done to the setup of your

computer via the Configuration Utility. The FLASH update process will

cause ALL SETTINGS ON ALL PAGES to revert back to their factory defaults.

You will need to restore the customizations in step 11 below.

3.  Plug in the AC adapter and make sure the battery is installed.  It

is important that the computer have BOTH sources of power available.

* If both AC and battery power is lost during the update, your computer

may have to be sent in for service.

* If your battery does not have enough charge to complete the update,

you will receive a message saying to wait awhile while the battery

is charged. Press enter to check the battery charge level again.

Once the battery has built enough charge, the program will continue.

(The battery will charge more quickly if you hit Fn-F4 to put the

computer into suspend. Hit Fn to bring it out of suspend. The battery

level must be in the green region of the battery gage for the FLASH

update to proceed.)

4.  Insert the FLASH Update Diskette into the diskette drive.

5.  Reboot the computer and allow it to boot from the FLASH update diskette.

6.  Follow the instructions on the screen.

7.  When you are told to press the power switch, push it then release it.

This will NOT power the system off, it will enable the Flashing mechanism

of the ROM.

9.  Continue following the prompts.  When you see a bar graph with "*"s,

do NOT touch anything, especially any power supplies.

10. When "Program completed successfully" appears, it is safe to turn

your computer OFF (by pressing and releasing the power switch).

Remove the diskette from the drive before turning the computer back

ON (the diskette is no longer needed).

11. At Power On, you will see error messages, but don't be concerned.

Press "ENTER", make any desired changes in the Configuration Utility,

press "ESC", and select "Yes" to save the Configuration. The next time

you boot your computer, these error messages should not appear.

12. The update of your ThinkPad 701 BIOS and Diagnostics is complete.